# Supplementary Information

When kinematic constraints conflict with rocket telemetry:
sparse trajectory reconstruction across two real-flight datasets

Kağan Yurtkölesi

Department of Aerospace Engineering, Izmir University of Economics, Izmir, Türkiye
Correspondence: kagan.yurtkolesi@std.iue.edu.tr

# S1. Overview and relation to the main article

This Supplementary Information (SI) documents data partitions, hyperparameter selection, deterministic sparse-observation masks, flight-level outcomes, targeted Iridium NEXT-5 diagnostics, the independent FlightSketch reconstruction, and integrity checks supporting the main article. It is intended to make the analysis auditable without expanding the claims beyond the evidence. The primary estimand throughout is paired performance under identical observations, model capacity, training protocol, and random seed; the penalty distinguishes the model objectives, while RQ1 final epoch counts were fixed separately by model selection.

The two research questions have different evidential roles. RQ1 is a split-preserving analysis of 40 Falcon 9 flights with an untouched six-flight test set. RQ2 is an independent, public-source reconstruction whose confirmatory v1.1 split was fixed after a source and implementation audit; it is not described as a pristine sealed test because the historical split artifact was unavailable and prior aggregate results had already been observed.

Table S1. Supplement map.

| Section | Contents |
| --- | --- |
| S2 | RQ1 source cohort and split roster |
| S3 | RQ1 model, loss, selection, and deterministic masking |
| S4 | RQ1 detailed outer-validation and held-out results |
| S5 | Iridium NEXT-5 diagnostics, ablations, and family transfer |
| S6 | RQ2 source audit and curation |
| S7 | RQ2 development analysis and confirmatory reconstruction v1.1 |
| S8 | Measurement-channel consistency audit |
| S9 | Reproducibility and integrity records |
| S10 | Data, code, permissions, and submission checklist |

# S2. RQ1 data provenance and cohort definition

RQ1 used 40 real Falcon 9 launch trajectories. Each flight was represented from T+0 through T+120 s on a 1 Hz grid (121 samples). The locked partition comprised 28 training flights, six outer-validation flights, and six held-out test flights. Within the 28-flight training partition, 22 flights were used for inner fitting and six for inner validation during model selection. Payload mass missingness was encoded explicitly rather than imputed silently.

Table S2. Complete RQ1 flight roster and locked outer split.

| Flight | Partition |
| --- | --- |
| ABS-2A | Training |
| Bangabandhu-1 | Training |
| BulgariaSat-1 | Training |
| EchoStar 23 | Outer validation |
| Formosat-5 | Training |
| GovSat-1 | Training |
| GPS III SV01 | Outer validation |
| Hispasat 30W-6 | Training |
| Inmarsat-5 F4 | Training |
| Iridium NEXT-1 | Training |
| Iridium NEXT-3 | Training |
| Iridium NEXT-4 | Training |
| Iridium NEXT-6 | Training |
| Iridium NEXT-8 | Training |
| Jason-3 | Outer validation |
| JCSAT-16 | Training |
| JCSAT-2B | Training |
| Koreasat-5A | Training |
| NROL-76 | Training |
| Orbcomm OG2 | Training |
| PAZ | Training |
| SES-10 | Training |
| SES-11 | Outer validation |
| SES-12 | Training |
| SpaceX CRS-10 | Training |
| SpaceX CRS-12 | Training |
| SpaceX CRS-13 | Training |
| SpaceX CRS-14 | Outer validation |
| SpaceX CRS-16 | Training |
| SpaceX CRS-8 | Outer validation |
| SpaceX CRS-9 | Training |
| TESS | Training |
| X-37B OTV-5 | Training |
| Zuma | Training |
| Intelsat 35e | Held-out test |
| Iridium NEXT-5 | Held-out test |
| SES-9 | Held-out test |
| SpaceX CRS-11 | Held-out test |
| SSO-A | Held-out test |
| Thaicom-8 | Held-out test |

The exact 22/6 inner-fit/inner-validation assignment is retained in the machine-readable workflow record; no held-out test flight entered tuning or model selection.

# S3. RQ1 model, selection, losses, and masks

## S3.1 Input representation and architecture

Both models received eight inputs: normalized time, standardized payload mass, a payload-missing indicator, and a five-level Block 1–5 one-hot representation. The shared multilayer perceptron contained two hidden layers of 32 tanh units. Outputs were standardized altitude and speed. Training used Adam; data scaling was estimated from fitting data only.

Table S3. RQ1 hyperparameter search and locked configuration.

| Element | Candidates / rule | Selected |
| --- | --- | --- |
| Learning rate | 0.0003, 0.001 | 0.001 |
| Coupled L2 weight decay | 0, 1e-6, 1e-5, 1e-4 | 1e-4 |
| Selection seeds | 11, 29, 47 | Same for paired models |
| Final evaluation seeds | 11, 29, 47, 71, 97 | Five paired seeds |
| Final training epochs | Selected without test access | NN: 560; KC-NN: 400 |
| Primary observation rate | Fixed before held-out evaluation | 5% |

## S3.2 Objective and physics-weight calibration

The data objective was the equal-weight mean of standardized altitude and speed mean-squared errors. Automatic differentiation supplied q = dĥ/dt. The one-sided kinematic penalty averaged three squared hinge terms: negative climb rate, climb rate exceeding predicted speed, and negative predicted speed. All velocity-like terms were normalized by the training speed scale. The fixed penalty multiplier λkin = 33.68225796 was obtained from the inverse median initial gradient-norm ratio (median ratio 0.02968922099) using training information only.

These inequalities encode consistency, not a complete flight-dynamics model. In particular, speed is not necessarily vertical velocity; the upper-bound term dĥ/dt ≤ v̂ is therefore the most directly defensible relation when v̂ represents speed magnitude. A penalty can regularize sparse interpolation while also degrading telemetry-channel fit when the measured channels, preprocessing, or model class do not satisfy the assumed relation exactly.

## S3.3 Sparse-observation protocol

For each flight and observation fraction, NN and KC-NN received exactly the same deterministic mask. Endpoints were retained. Interior samples were selected from a fixed hash-derived ordering until the requested count was reached, with at least two observations per trajectory. Primary inference used 5% observations; 10%, 25%, 50%, and 100% were sensitivity analyses. Scores were computed on the complete trajectory, not only on observed points. This section describes RQ1 training-target masking. RQ2 v1.1 applies sparsity to reconstruction inputs but trains against complete fitting trajectories, as clarified in S7.

# S4. RQ1 detailed results

## S4.1 Primary held-out test at 5% observation

Table S4. RQ1 held-out test results at 5% observation.

| Metric | NN | KC-NN | Paired interpretation |
| --- | --- | --- | --- |
| Altitude RMSE (m) | 2304.55 | 1920.30 | Lower for KC-NN |
| Speed RMSE (m/s) | 46.77 | 69.87 | Lower for NN |
| Kinematic violation fraction | 0.4262 | 0.0612 | Lower for KC-NN |
| Standardized composite difference | — | +0.018634 | Positive = KC-NN worse |
| Flight-level wins | — | 1 of 6 | KC-NN |
| Flight bootstrap 95% interval | — | [-0.020757, +0.045011] | Includes zero |

The constraint greatly reduced inequality violations and improved altitude reconstruction on average, but the speed penalty outweighed that benefit in the prespecified composite. The bootstrap interval and one-of-six flight win rate do not support a general held-out advantage.

Table S5. RQ1 held-out test flight-level composite scores at 5%.

| Flight | NN | KC-NN | KC-NN − NN | Winner |
| --- | --- | --- | --- | --- |
| Intelsat 35e | 0.113894 | 0.161973 | +0.048078 | NN |
| Iridium NEXT-5 | 0.224670 | 0.149446 | -0.075225 | KC-NN |
| SES-9 | 0.338075 | 0.393535 | +0.055460 | NN |
| SpaceX CRS-11 | 0.150736 | 0.171240 | +0.020504 | NN |
| SSO-A | 0.155988 | 0.188540 | +0.032552 | NN |
| Thaicom-8 | 0.076771 | 0.107207 | +0.030437 | NN |

Lower is better. Small rounding differences can occur when recomputing displayed differences from rounded component scores.

Table S6. RQ1 outer-validation flight-level composite scores at 5%.

| Flight | NN | KC-NN | KC-NN − NN |
| --- | --- | --- | --- |
| EchoStar 23 | 0.111567 | 0.138696 | +0.027128 |
| GPS III SV01 | 0.098951 | 0.106918 | +0.007967 |
| Jason-3 | 0.514890 | 0.528352 | +0.013463 |
| SES-11 | 0.122906 | 0.153247 | +0.030341 |
| SpaceX CRS-14 | 0.182414 | 0.200935 | +0.018521 |
| SpaceX CRS-8 | 0.176660 | 0.179747 | +0.003088 |

Mean paired difference = +0.016751; flight-bootstrap 95% interval [0.009015, 0.024728]; KC-NN wins 0/6 flights.

Table S7. RQ1 observation-rate sensitivity.

| Partition | Obs. | Alt. NN | Alt. KC | Speed NN | Speed KC | Composite Δ |
| --- | --- | --- | --- | --- | --- | --- |
| Test | 100% | 1506.34 | 1360.63 | 35.65 | 39.34 | -0.000832 |
| Test | 50% | 1465.44 | 1344.74 | 33.42 | 38.85 | +0.003014 |
| Test | 25% | 1537.97 | 1440.71 | 37.05 | 41.43 | +0.002440 |
| Test | 10% | 1710.03 | 1614.37 | 38.43 | 45.17 | +0.006180 |
| Test | 5% | 2304.55 | 1920.30 | 46.77 | 69.87 | +0.018634 |
| Outer validation | 100% | 1247.77 | 1137.64 | 59.65 | 62.83 | -0.000013 |
| Outer validation | 50% | 1271.91 | 1156.97 | 59.75 | 63.28 | +0.000307 |
| Outer validation | 25% | 1290.57 | 1232.63 | 61.16 | 62.69 | -0.000238 |
| Outer validation | 10% | 1485.96 | 1421.62 | 63.90 | 67.48 | +0.002674 |
| Outer validation | 5% | 2148.51 | 1773.23 | 67.05 | 88.68 | +0.016751 |

Altitude is in metres; speed is in m/s; Δ = KC-NN − NN, so negative favors KC-NN.

# S5. Iridium NEXT-5 targeted diagnostic

Iridium NEXT-5 was the sole held-out flight on which KC-NN improved the 5% composite score. This section tests explanations suggested by that result without promoting a post hoc subgroup finding to a general causal conclusion. At 5%, altitude RMSE decreased from 2260.3 to 933.8 m, speed RMSE decreased from 78.88 to 68.96 m/s, and the kinematic-violation fraction decreased from 38.0% to 5.8%. KC-NN won the composite for all five paired seeds; the altitude term accounted for approximately 79.5% of the composite improvement. The unconstrained prediction showed its largest altitude overshoot after roughly 30 s, especially during 61–90 s.

Table S8. Iridium NEXT-5 observation-rate sensitivity.

| Observation rate | NN | KC-NN | KC-NN − NN |
| --- | --- | --- | --- |
| 100% | 0.1641 | 0.1046 | -0.0595 |
| 50% | 0.1378 | 0.0930 | -0.0448 |
| 25% | 0.1689 | 0.1130 | -0.0559 |
| 10% | 0.1974 | 0.1311 | -0.0663 |
| 5% | 0.2247 | 0.1494 | -0.0752 |

The flight-specific advantage persists across all tested observation rates and is largest under the primary 5% condition.

Table S9. Iridium NEXT-5 constraint ablation at 5%.

| Model / ablation | Composite | Interpretation |
| --- | --- | --- |
| Unconstrained NN | 0.2247 | Reference |
| Full KC-NN | 0.1494 | All three inequalities |
| Omit dĥ/dt ≥ 0 | 0.1385 | Removing monotonic-altitude term improves score |
| Omit dĥ/dt ≤ v̂ | 0.3210 | Upper-bound term is essential on this flight |
| Omit v̂ ≥ 0 | 0.1494 | No material effect |

Table S10. Leave-one-flight-out Iridium family transfer at 5%.

| Held-out | NN | Full KC | Upper only | Upper − NN | Winner |
| --- | --- | --- | --- | --- | --- |
| NEXT-1 | 0.0960 | 0.1251 | 0.1129 | +0.0169 | NN |
| NEXT-3 | 0.1740 | 0.2120 | 0.2111 | +0.0371 | NN |
| NEXT-4 | 0.2654 | 0.2194 | 0.2080 | -0.0573 | Upper-bound |
| NEXT-6 | 0.1180 | 0.1455 | 0.1326 | +0.0146 | NN |
| NEXT-8 | 0.2169 | 0.1818 | 0.1753 | -0.0416 | Upper-bound |

The prespecified family criterion required a negative mean difference and at least 4/5 upper-bound wins. It failed: 2/5 wins; mean upper-bound − NN = −0.0061, driven by NEXT-4 and NEXT-8.

Interpretation: NEXT-5 benefits because the upper-bound relation specifically suppresses a sparse-data altitude overshoot that is not corrected reliably by the unconstrained network. The ablation localizes the gain to dĥ/dt ≤ v̂, while the leave-one-flight-out family experiment shows that this mechanism does not transfer uniformly even within nominally similar missions. Thus “why it works” is supported as a flight-level mechanism, not established as a mission-family causal law.

# S6. RQ2 source audit and curation

The independent source audit indexed 4,040 public files, identified 617 candidate flight files, and retained 614 technical passes after parsing and quality checks. These represented 68 provisional rocket/group labels. Four uploaders contributed retained files; the dominant uploader supplied 586/614 (95.4%), creating a material contributor-concentration limitation. Of the retained files, 482 contained acceleration in addition to pressure/altitude/velocity channels and 132 contained pressure/altitude/velocity only.

Table S11. RQ2 source-audit summary.

| Quantity | Value |
| --- | --- |
| Public index entries | 4,040 |
| Candidate flight files | 617 |
| Technical passes | 614 |
| Provisional groups | 68 |
| Distinct uploaders | 4 |
| Dominant-uploader contribution | 586/614 (95.4%) |
| Acceleration-equipped files | 482 |
| Pressure/altitude/velocity-only files | 132 |
| Median raw sampling interval | 0.020 s |
| Median duration | 50.101 s |
| Median altitude span | 611.074 ft |
| Median maximum positive velocity | 212.505 ft/s |

Automated pass status establishes parsability and minimum channel coverage, not provenance equivalence, calibration validity, independence, or aerodynamic comparability. Provisional group labels were normalized from source metadata and should not be read as certified vehicle identities. Contributor concentration limits claims about population-level generalization.

# S7. RQ2 development analysis and reconstruction v1.1

## S7.1 Historical development analysis

The historical development analysis used 335 flights and 17,864 one-hertz samples across 41 provisional groups: 33 groups/257 flights for training and eight groups/78 flights for outer validation. At the primary 5% observation rate, the mean standardized composite was 0.3479 for NN and 0.3449 for KC-NN, with KC-NN winning 3/5 paired seeds. At 10%, 25%, 50%, and 100%, KC-NN lost all 20 seed comparisons. These development outcomes motivated a stricter independent reconstruction but are not pooled with v1.1 confirmatory estimates.

## S7.2 Confirmatory reconstruction v1.1

Version 1.1 used 363 fitting flights from 31 groups, 75 inner-validation flights from six groups, and 46 held-out flights from eight groups. Both models used two 32-unit tanh layers, fitting-only scaling, Adam with learning rate 0.001, coupled L2 weight decay 0.0001, a maximum of 3,000 epochs, and patience 250. Seeds were 11, 29, 47, 71, and 97. The physics weight was fixed at 1.91. The primary 5% mask retained endpoints and then followed a deterministic SHA-derived ordering until max(2, ceil(0.05n)) observations were selected. Sparse observations are used to construct input features. The stack and loss_grad functions retain all fitting targets, and early stopping uses all inner-validation targets. This is supervised sparse-input reconstruction, not training with only 5% of fitting labels.

RQ2 v1.1 fits dataset-specific weights rather than transferring RQ1 checkpoints. The archived residual uses finite differences of centered altitude and the trapezoidal mean of centered velocity, divided by the fitting velocity scale. Altitude centering cancels, but velocity centering produces an offset of μ_v/σ_v = −0.0058894643/14.3564722162 ≈ −0.00041023 from the physical-channel equality residual. Reported scores remain those of this archived implementation. No corrected-residual retraining is claimed. This distinction qualifies any mechanistic attribution of the RQ2 comparison to exact physical equality.

Table S12. RQ2 reconstruction v1.1 partition.

| Partition | Groups | Flights | Role |
| --- | --- | --- | --- |
| Fit | 31 | 363 | Parameter estimation |
| Inner validation | 6 | 75 | Stopping/model control |
| Held out | 8 | 46 | Paired final reconstruction |

Table S13. RQ2 reconstruction v1.1 held-out results at 5%.

| Metric | NN mean ± SD | KC-NN mean ± SD | Result |
| --- | --- | --- | --- |
| Altitude RMSE | 44.73 ± 3.63 | 57.52 ± 3.46 | NN lower |
| Speed RMSE | 10.14 ± 0.23 | 11.04 ± 0.77 | NN lower |
| Standardized composite | 0.6096 ± 0.0274 | 0.7145 ± 0.0369 | NN lower |
| Paired composite difference | — | +0.1049 ± 0.0558 | Positive = KC-NN worse |
| Seed-level wins | — | 0/5 | KC-NN |

The ± value for the paired difference is the sample standard deviation across the five paired seeds, not a confidence interval.

Table S14. RQ2 held-out groups in reconstruction v1.1.

| Group | Flights | Direction of mean paired difference |
| --- | --- | --- |
| Big Bessie | 6 | KC-NN mean lower; uncertainty crosses zero |
| Aphelion 2 | 5 | NN lower |
| Zenith II | 6 | NN lower |
| Trident 2 | 6 | NN lower |
| Super Mars Snooper | 5 | NN lower |
| Sterling Silver (single stage) | 5 | NN lower |
| Orbitron | 6 | NN lower |
| Nova Payloader 256 | 7 | NN lower |

Seven of eight groups favored NN. Big Bessie was the only negative group mean, and its uncertainty crossed zero.

Reconstruction v1.1 corrected optimizer conformance and excluded the earlier AdamW diagnostic from confirmatory inference. It is labeled “confirmatory reconstruction” because the implementation and split were fixed before v1.1 execution, but it is not a pristine sealed test: the historical 41-group split artifact was unavailable and prior aggregate outcomes had already been exposed.

# S8. Measurement-channel consistency audit

Across the 335-flight historical development corpus, the per-flight root-mean-square mismatch between finite-difference altitude rate and the supplied velocity channel averaged 5.55 m/s (median 4.50 m/s). This statistic is a magnitude measure, not a signed bias, and it does not identify a unique cause. Plausible contributors include speed-versus-vertical-velocity semantics, independent filtering, timing offsets, quantization, pressure-altitude conversion, sensor noise, and genuinely nonvertical motion.

The mismatch audit is consistent with channel inconsistency as a possible contributor to the unfavorable penalty result, but it does not isolate that cause. The archived residual also has the centering offset documented in S7, and the two datasets use different observation and supervision protocols. The evidence therefore supports cautious evaluation of channel-coupling penalties, not a controlled cross-dataset causal comparison.

# S9. Reproducibility and integrity records

Table S15. Frozen workflow fingerprints and replay checks.

| Artifact / run family | Integrity record | Verification |
| --- | --- | --- |
| RQ1 locked manifest | 89728925fd31f436dbb9438556157e7a030ce7ee881b557cdad434257b2fa64b | 98 main runs completed/replayed |
| Iridium family manifest | 6a536dcb970f45ceac5987fa2e4f29ef006d9d3dedb7e3263dc1d93a85028813 | 75 runs completed/replayed |
| Iridium mask record | 984d0f748060ca8f58d1da3cb8084cad0dd56f63442dc86b91fab6d99abfe8a9 | No original NEXT-5 test trajectories used |
| RQ2 reconstruction v1.1 | 9efc8c68fc63ac7603e471e101571417017ac5bfb51398fa2f718396761f8782 | 10 checkpoints replayed within 1e-12 |

A reproducible run record should include source-file identifiers, split membership, preprocessing configuration, mask hash, seed, optimizer settings, stopping epoch, model checkpoint hash, software environment, and final metrics. Fingerprints above identify the frozen analysis records used for the article; they are not substitutes for depositing the corresponding machine-readable files.

# S10. Data, code, sources, and submission checklist

## S10.1 Data provenance and sharing boundary

RQ1 results derive from the archived trajectory cohort and split records described above. RQ2 used publicly accessible FlightSketch flight pages and downloadable CSV files. Public download access supports inspection and analysis but does not by itself establish a license to republish the raw files. Accordingly, this SI reports derived metadata, aggregate statistics, and integrity identifiers; the third-party raw CSV files are not included in the package.

## S10.2 Public code and archival identifiers

The public version 0.9.0 reproducibility archive is https://doi.org/10.5281/zenodo.23080487. The maintained repository is https://github.com/kaganyurtk/sparse-trajectory-reconstruction. The archived files are a frozen snapshot; this submission supplement updates availability and implementation descriptions without modifying archived numerical records.

FlightSketch and its flight-log contributors are acknowledged as the data sources. The public flight index is https://flightsketch.com/flight-filter/ and individual records are available at https://flightsketch.com/flights/<id>/. Raw telemetry is not redistributed in the package.

Recovered_Experiment_Records_v0.9.0.zip contains the original RQ1 final package, NEXT-5 ablation and Iridium-family packages, and RQ2 reconstruction v1.1 split, manifest, checkpoints and historical verification records. The RQ1 inner split matches the deposited implementation.

The repository software and documentation licenses do not extend to third-party telemetry. Source attribution and any applicable source terms continue to apply to the underlying data.

RECOVERED_RECORDS_VERIFICATION.json records the archive-integrity and split-consistency checks performed during package recovery. These checks do not constitute a new training run or metric replay. The later OFFLINE_REPLAY_AUDIT_2026-10-01.md recomputes saved RQ1 prediction metrics and RQ2 summary arithmetic without a fresh raw-telemetry replay.

The deposited historical records retain the analysis fingerprints in Table S15. Running the recovered workflows requires permitted source data and configuration of historical paths. The assembled package has not yet undergone a clean-environment end-to-end replay.